Subnetting Plan: 192.168.1.0/24

- Goal: Create 3 subnets for departmental VLANs (Admin/IT, Finance/HR, and Customer Service).

- Formula: subnets. Borrowing 2 bits () provides four subnets, satisfying the requirement.

- New Prefix: /24 + 2 = /26.

- New Subnet Mask: 255.255.255.192 (11111111.11111111.11111111.11000000).

- Magic Number: (block size).

Departmental Subnet Breakdown

| Department | VLAN | Network ID | Valid Host Range | Broadcast ID |
| --- | --- | --- | --- | --- |
| Admin/IT | 10 | 192.168.1.0 | 192.168.1.1 – 192.168.1.62 | 192.168.1.63 |
| Finance/HR | 20 | 192.168.1.64 | 192.168.1.65 – 192.168.1.126 | 192.168.1.127 |
| Customer Svc | 30 | 192.168.1.128 | 192.168.1.129 – 192.168.1.190 | 192.168.1.191 |

Implementation Requirements Checklist

- DHCP: The router will serve as the DHCP server for each VLAN using the DORA process (Discover, Offer, Request, Acknowledgement) to assign addresses automatically.

- Connectivity: Since departments are in different VLANs (broadcast domains), you must implement inter-VLAN routing (likely "router-on-a-stick") to allow them to communicate.

- Wireless: Each department will connect to the switch via an Access Point configured with the corresponding departmental SSID.

This scheme provides 62 usable host addresses per department, which is ideal for a small branch office.