Subnetting Plan: 192.168.40.0/24

- Goal: Create 2 subnets for two departments (Accounts and Printing).

- Device Count: Each department requires 3 hosts (2 PCs + 1 printer), well within the capacity of this scheme.

- Formula: . To get 2 subnets, you must borrow 1 bit ().

- New Prefix: /24 (original) + 1 (borrowed) = /25.

Subnet Mask Calculation

- Binary Mask: 11111111. 11111111. 11111111. 10000000.

- Decimal Mask: 255.255.255.128.

- Magic Number: . This determines the "block size," meaning your networks will increment by 128 in the fourth octet.

Departmental Subnet Breakdown

| Department | Network ID | Valid Host Range | Broadcast ID |
| --- | --- | --- | --- |
| Accounts | 192.168.40.0 | 192.168.40.1 – 192.168.40.126 | 192.168.40.127 |
| Printing | 192.168.40.128 | 192.168.40.129 – 192.168.40.254 | 192.168.40.255 |